Crestron

For more than 30 years, Crestron has been the world's leading manufacturer of advanced control and automation systems. We offer total solutions using Crestron products for integration and control of audio, video, computer, the Internet and more.

Crestron's leadership comes from a total focus and commitment to providing the best products, programs and services in the industry-all backed by Crestron's dedicated team of people.

While Crestron products are flourishing in businesses and connected homes worldwide, they're constantly at work developing new generations of Crestron products. Crestron has the vision to imagine the future and make it happen today.

In addition to World Headquarters in Rockleigh, New Jersey, Crestron has sales and support offices throughout the U.S., Canada, Europe, Asia, Latin America and Australia. Regardless of where they are in the world, every Crestron office is prepared to always deliver products and services on time, every time.

For more than 30 years, Crestron has been the world's leading manufacturer of advanced control and automation systems. We offer total solutions for integration and control of audio, video, computer, the Internet and more.

Our leadership comes from a total focus and commitment to providing the best products, programs and services in the industry - all backed by Crestron's dedicated team of people. Products Plus People - that's what Crestron is all about.

Products... Innovative technology is the cornerstone of Crestron. Since the beginning, we've been a pioneer in the A/V industry, introducing a number of firsts. We were the first developers of microprocessor-based control systems, and were first to bring to the industry touchpanel technology for remote control applications. Crestron was also first-to-market with e-Control™, a truly viable Internet-based control solution that is now the industry standard for harnessing the power of the Web to control the A/V and environmental systems of businesses and homes around the globe.

While our products flourish in businesses and connected homes worldwide, we're constantly at work developing new generations of Crestron products. We have the vision to imagine the future and make it happen today.

Plus... Symbolic of the services and programs that support Crestron products. Our I2P Partnership Program, the Crestron Technical Institute, Crestron Authorized Independent Programmers and, of course, True Blue Support all demonstrate our total commitment to our industry, and to our customers.

People. Crestron people make the difference. Our people commit themselves to deliver unwavering support, turning problems into solutions. They care about our customers, and are dedicated to proving it each and every day.

In addition to our World Headquarters in Rockleigh, New Jersey, Crestron has sales and support offices throughout the U.S., Canada, Europe, Asia, Latin America and Australia. Regardless of where they are in the world, every Crestron office is prepared to always deliver products and services on time, every time.

------------------------

Panasonic Phone/Intercom Systems

Today many homeowners are having additional lines installed for modems, fax machines, home offices or for teenagers who simply "must have" their own line.

The Panasonic KXTA Advanced Hybrid Telephone System offers a host of time saving features designed to streamline communications within the home and better manage outside lines as well.  Residential, home office, fax and internet lines are accessible from any phone in the home or home office.

The optional door intercom allows up to four door intercoms to be added to the system.  When the button on the door intercom is pushed, the system will ring only those phones which you've selected, and when answered, will establish a two-way conversation with whomever is at the doorphone.  Each door intercom can have a unique ringing pattern allowing you to distinguish which door intercom was pressed.

A variety of proprietary system phones with flexible line buttons enable you to tailor the system to your exact needs within your home.  Call other extensions, dial frequently called numbers, or page all proprietary phones with the touch of one button.

To further increase freedom and mobility throughout your home,  Panasonic offers the KX-T7885 series cordless telephone.  It operates on 900 Mhz frequency for superior audio clarity, extended range and freedom from interference.  It allows access to most of the features found on the corded proprietary system phones and has three illuminated line buttons.  Dial or page other extensions, quickly put calls on hold and, because of the multiple line buttons, easily determine which line is ringing.  The compact design of this cordless wonder makes it small enough to fit in your pocket or wear on your belt - and it comes with a separate charging unit that allows you to recharge the handset anywhere there's an electrical outlet.

The Panasonic KX-TA system is available in customizable sizes to fit almost any home.  It is initially configured with 3 central office lines and 8 extensions and expands to 6 CO lines and 24 extensions.  Each system has a host of user programmable software functions to help streamline internal and external communications.  The Panasonic KX-TA system can accomodate almost any device that operates on conventional telephone lines without any additional hardware.  A full line of KX-T7000 series proprietary telephones allow you to custom design a system that's right for you and your budget as well.  Whether you are building a new home, opening a home office or just looking to improve your current home communication system - for flexibility, convenience and features, the Panasonic KX-TA Advanced Hybrid Telephone System is the obvious choice.

Custom Installation 1116.1-1.1

WHAT IS "CUSTOM INSTALLATION" ?

Just what is custom installation? The "custom" industry consists of skilled professionals who design and install audio/video and other related systems in residential and commercial spaces. These systems differ from what you would buy in a retail store in that they are engineered to not only meet and fulfill the needs of the client, but to optimize performance while complementing the physical and aesthetic demands of the space — hence the term "custom". These systems can be configured to accommodate security, telephone, and other home systems while providing a user-friendly interface for the customer.

Professional installers will typically work with the client to coordinate their efforts with other contractors — such as architects, interior designers, and electricians — to ensure that the system blends into the home both functionally and aesthetically.

A simple comparison: Whereas an off-the-shelf stereo system sits obtrusively in one's living room and serves only that space, a custom A/V system uses these components plus others to form a nearly invisible system providing music and video to any number of rooms.

There are many reasons why the business of custom installation is attracting such avid consumer interest. Today's audio and video components have become increasingly sophisticated and capable, yet one is often drawn to discount "superstores" when lured by low prices and a vast product selection. Unfortunately, many of these "superstores" lack the expertise of professional installers, and are therefore unable to provide quality service and advice, often to the customer's disadvantage. Where does one look to obtain good advice and make educated purchases of these products? More and more, consumers are turning to custom installers who can help them explore the myriad of home entertainment options now available to them, and explain the features and functions of each in laymen's terms. One of the best places to locate a qualified custom installer is CEDIA, which stands for Custom Electronics Design and Installation Association. They can be contacted at 1 (800) 669-5329.

Another factor which is contributing to the popularity of custom installation is that the consumer electronics market is driven by clever product applications, clearly the domain of the custom installer and manufacturers such as Niles.

A third reason for the growth of custom installation is equally significant — the fact that it caters to you, the homeowner, who dreads the complexity and unsightliness of traditional audio and video systems, yet wants to enjoy music throughout your home. In the capable hands of a custom installer, special devices like those supplied by Niles can make the most sophisticated audio/video systems easy to use and practically invisible.

A fourth reason why custom installation has become so popular is that it is now being fed by its own prominence. The growing popularity of "home theaters" (which were pioneered by custom installers) speaks to a change in the way you look at audio and video components. Many see them as an integral part of one's home and lifestyle, and as an investment in both. Manufacturers of traditional components such as big-screen TV sets and audio/video components are targeting these new sensibilities by promoting professionally installed home theaters and "multi-room" audio systems.

But perhaps the most compelling reason for the continuing success of the custom installation industry is right before your eyes. Your home is rapidly becoming a monument to your lifestyle. If you exercise regularly, you want a home gym. If you work a lot of overtime, you want a home office. If you enjoy watching movies, you want a home theater. "Convenience" is no longer simply a goal, it's a must. That alone describes the design objective in home entertainment electronics, one which ensures the continued growth of the custom installation industry in the coming days as High-Definition TV, DVD, and a litany of other home automation technologies as they become available.

THX Home Program

Home THX Audio Systems, developed by THX Ltd., have become the world's standard for film enjoyment in the home. Now in its 15th year, THX licenses its patented technology to leading A/V and PC manufacturers worldwide. The legendary training program has certified more than 1300 dealers as among the highest qualified to sell and install Home THX products. THX also certifies home video software through the THX Digital Mastering Program, ensuring that DVD software, laser discs and VHS videotapes provide the best image and sound quality possible through the use of patented THX technology and our unique quality control expertise.

Home THX Program Overview

In 1985, THX Ltd. began researching the reproduction of movie soundtracks in the home. Hollywood had announced that more people were watching movies on video than in movie theatres. Was it possible to get the same experience at home? THX engineers assembled a high performance home theatre sound consisting of matched high-end speakers, a Dolby Pro Logic decoder, high powered amplifiers, and a laser disc player. They listened to familiar soundtracks on the system and compared the quality of that sound to the mixing theatres. To their surprise, the home system sounded very different.

The difference in sound quality and character between the Home Theatre and the Mixing Stage was surprising. Home audio components are generally of very high quality and precision. What was happening?

The Translation From Cinema to Home

Movies are mixed in movie theatres called dubbing stages over equipment and under standards carefully specified. The soundtracks created are designed to be played back in movie theatres using the same type of equipment and under the same standards. No wonder it was difficult to get the same sound over the test hi-fi system!

Home THX was created to solve this problem. First, circuitry had to be designed to accurately translate the sound from the movie environment into the home, correcting the tonal and spatial errors that occurred. Second, refinements in speaker designs had to be researched since most hi-fi equipment is designed to reproduce only 2 channels of sound in a home, and not the 4, 5 or more channels present in a movie soundtrack.

Marantz

For more than half a century, the name Marantz has identified the best in home entertainment. Today, even in the midst of burgeoning and often confusing technology, these components still emulate the vision that originally drove Saul Marantz to expand the sensory horizons of even the most demanding aficionados.

More than any other person, Saul Marantz defined premium home entertainment. Driven by his passion for music and his accomplishments as a classical guitarist he was never satisfied with the “hi fi” equipment of his day. So he built better; first in his basement, later in a factory. His talent for industrial design and his ability to infuse talented engineers like Sidney Smith, and others with his vision, resulted in legendary products. This ensured that his company would remain a premier name in the industry he helped establish.

Audio One proudly carries Marantz products

Installation Services

System Engineering

Definitive Audio's in house engineering services and design documents benefit your project in several ways.  Through the engineering design process, we are able to consider your goals, the environment that the system will reside in, the equipment's performance, as well as consider other aspects of the project to develop a comprehensive, performance-oriented solution.  This process is used internally to help finalize the system design and equipment selection, and our installation staff refers to the engineering design documents to insure that the final installation is consistent with the design intent.  Definitive Audio will provide copies of these documents to you and when requested, to the appropriate trades on the project to allow them to successfully coordinate their work with the Audio/Visual System.

Retrofit Installation

Retrofitting an Audio/Visual System into your existing home allows us to integrate the system into your living environment in the least obtrusive and most aesthetically pleasing manner.  The required cables are pulled through your existing walls and then Speakers, Keypads, Volume Controls, etc. are installed.  We attempt to utilize attic or crawl spaces to create the most efficient cable runs, however existing conditions often require our installation staff to arrive at other creative solutions.

We then deliver, install and connect the remaining items and troubleshoot bugs in the finished system.  We prefer to deliver and set-up the electronics in a clean and secure environment, when all the other contractors on the project are finished.

The installation labor and materials cost estimates indicated within this Project Proposal are based upon expected construction and structural conditions, which cannot be verified until the Retro-Fit installation has begun and existing conditions revealed.  For this reason, additional installation labor and materials may be required to complete the project.  Our installation staff is highly skilled and experienced and every effort is made to complete your project according to our cost estimates in a manner that will minimize any necessary "Related Work" required at the completion of the installation.

New Construction Installation

New construction projects typically require that your Audio/Visual System be installed in several separate phases.  The Pre-Wire Phase is done immediately after the electrical rough in, yet prior to installation of insulation and drywall.  This phase consists of the installation of cut-in boxes, speaker brackets, cables and wires with the appropriate home runs to the Main Equipment Center (MEC).

The Trim Phase is when we deliver and install flush mounted speakers, volume controls, keypads and other built-in controls.  The Trim Phase is ideally done after primer/painting, and before wall papering or flooring.

During the Final Installation Phase, the remaining equipment and electronics are delivered to your home.  At this time, we will install and connect the remaining items and troubleshoot bugs in the finished system. We prefer to deliver and set-up the electronics in a clean and secure environment, when you are actually occupying the space and all the other contractors on the project are finished.

Related Work

"Related Work" such as: structural framing, high-voltage electrical services, telecommunication services, patching & repairing drywall or other existing surfaces, painting, cabinet making and finish carpentry, etc., is often necessary to carry out the implementation of an Audio/Video System.  "Related Work" and the costs to perform said work, are not included in this Project Proposal.  When requested, Definitive Audio will provide information related to the Audio/Video System, to the appropriate trade on the project to allow them to coordinate their "Related Work" with the Audio/Visual System.

Existing Conditions Work, Equipment & Systems

When requested, Definitive Audio will attempt to incorporate your existing Audio/Visual Equipment whenever possible.  Due to technological changes inherent in the consumer electronics industry, we cannot guaranty compatibility of your existing Audio/Visual Equipment with the proposed Audio/Video System.  Often times it is wise to replace this equipment, as opposed to modifying your new system to accommodate old technology.

Definitive Audio makes no claims and does not warrant any existing equipment, work or systems, provided, performed or installed by others.  Corrections, replacements, modifications and or troubleshooting of these items, if required for proper operation of the completed Audio/Video System, will be brought to the client's attention.  Definitive Audio cannot be held accountable for delays caused by such items.  If the client desires Definitive Audio to make the necessary corrections to existing conditions, the client then agrees to pay Definitive Audio for the labor and materials required to complete the necessary corrections, at the labor rates and materials costs current at the time the work is performed.  These costs would be in excess of the costs estimated in this Project Proposal.  Agreement to perform this work is at Definitive Audio's discretion.

System Tests and Training

Upon completion of the system installation, and before demonstration, the system will be adjusted for optimum performance and tested to insure that the system components are performing within the manufacturer's specifications and within the specified design intent.  Your Definitive Audio Sales Associate will then train you on the use and operation of your system.  At this time, you will also be provided with the owner's manuals and warranty registration cards included with the installed equipment.Note: Unless indicated otherwise, Labor and Materials costs identified in this Project Proposal are estimates only.  The client shall be billed for actual Labor and Materials required to complete the project at the labor rates and materials costs as specified in the Custom System Installation Contract.

Control Systems

How you and your family interact with your home entertainment system largely affects your enjoyment of the system.  It has been our experience that client's who invest in a control system, enjoy using their entertainment system more, use them more frequently and have family and guests that are able to use the system easily.

The intent of Definitive Audio's control systems design & programming services is to create for our clients a friendly interface with their Audio Video system that allows everyone in the family to easily operate the audio visual system.  In addition to creating an interface that is straightforward to use, we provide access to additional control of some of the more advanced and less frequently used functionality offered by the equipment.  All audio video equipment is unique; therefore, different products will have varying levels of control options.

Benefits

q Simplified control of your compatible audio-visual equipment.  You do not need to know the details of how all the equipment works to work the system.

q Multiple commands initiated at the touch of one button.  Powering on each piece of equipment, selecting the correct source input and surround mode on the surround processor, selecting the proper input on the video processor and projector or TV, as well as adjusting your lights to your favorite level and closing your motorized blinds, can now be achieved through the press of a single button.

q Touchpanel designs that are personalized with your name, and incorporate commonly used commands that are easily accessed from where you need them.

q Each button is specific to the device and function that it controls.  The confusion generated by universal remote controls, which frequently have one-button control different functions on different devices, is eliminated.

q The control systems that we specify utilize RF (Radio Frequency) transmitters and receivers to provide reliable communication without having "to point the remote" in a certain direction.  In addition, RF allows communication to occur through obstacles such as furniture, cabinets and even operate through walls.

q The control systems that we specify incorporate an open architecture, which easily provides for expansion and changes to your system.

q Two-Way Communication systems offer feedback to your touchpanel to assist you in determining the status of your system.  Current Source Selection, Volume Level, Surround Mode and other items can all be indicated on the touchpanel's display.

Distributed Audio Systems

Whether you're entertaining guests, reading the Sunday paper, relaxing by the pool, or catching up on some work, it is more enjoyable when accompanied by your favorite music while at home.  Distributed Audio Systems provide this experience throughout your home in an attractive and easy to use manner.

Benefits

q     Easy to use, High performance Audio throughout your home, in an unobtrusive presentation

q     Multiple source components can be shared throughout your home (i.e. High Capacity CD Changer, AM/FM Tuner, DSS Audio, VCR Audio, etc.)  These shared sources can be listened to in different zones simultaneously with independent volume control in each room

q     Distributed Audio System Speakers can be shared by a Sub-Stereo Location (SSL) and switched between the two systems automatically

q     Water-resistant speakers and outdoor speakers can be incorporated into the system to provide music in damp locations (i.e. bathrooms and spas) or outdoors

q     An extra CAT5 run is installed to all VC locations to allow future upgrades to keypads

Control

Volume Control (VC) - VC's are the first control level available for Distributed Audio Systems.  VC's allow adjustment of the volume level of all speakers connected to the VC and also provide a means to turn off the audio being supplied to those speakers.

Keypad (KP) - KP's provide enhanced control of Distributed Audio Systems.  KP's can control volume levels, allow remote switching between source components (i.e. Tuner to CD), and often provide common control options (i.e. play, pause, stop, next track, next station and station presets) for compatible source components connected to the system.

Infra-Red Control (IR) - Some Distributed Audio Systems provide IR Receivers that allow the user to utilize similar functionality as a Keypad, via a handheld remote control.  Other Distributed Audio Systems offer a more elaborate IR repeater system that will accept and transmit any IR command to the system.

The best control solutions utilize a combination of the above systems, providing greater control options only where needed.  Increased control flexibility greatly reduces trips to the main equipment location.

Appearance

In-wall speakers can be "flush mounted" into walls or ceilings.  When the grilles for these speakers are painted, they blend in with even the most elegant décor.  We can work with you to locate equipment to optimize the performance of the system and preserve the appearance of the room.  This is best accomplished during the initial walk-thru of the project.  Keypads, Volume Controls and IR receivers are available in a variety of colors in the attractive "decora" style.  In addition, the main equipment for the Distributed Audio System; the controller, amplifiers and source components, can be located remotely, either in another room, in a closet or cabinet, or a main equipment room.

Home Theater System

Many considerations must be balanced when designing, specifying and installing your home theater system.  Our goal is to work with you to identify what your priorities are and help develop a solution that addresses those priorities.  Some considerations and our recommended solutions are listed below:

Performance

During the installation of your audio/visual system, Definitive Audio does many things to optimize the performance of your equipment for your particular installation. We locate speakers within your home for maximum performance (structural conditions allowing and assuming the location is to your liking).  We calibrate your surround sound processor by entering information into it regarding the types of speakers you are using, where they are located in the room, and then adjust their levels using a sound pressure level meter.  These steps insure that all of your speakers reproduce sound at the same relative loudness and that the sound from them is accurately timed.  We adjust the contrast of your display device and adjust the user-accessed convergence, allowing a sharper image and more accurate color reproduction in your video.

Appearance

We pride ourselves on the appearance of our Definitive Audio installed systems.  We strive to make sure that visible, and often times non-visible, wiring is done neatly.  We locate infrared emitters and receivers discretely whenever possible.  We make sure that equipment at the Main Equipment Center is aligned and all in-wall devices appear level and flush.  During your installation our installation staff will wear shoe covers, carry a drop-blanket and a waste receptacle, working cleanly to be respectful of your home and belongings.

Upgrades and future compatibility

Many of the products we carry and specify for a project have software that can be upgraded or are modular in design, allowing hardware or software improvements when provided by the manufacturer.  In addition, we try to specify systems anticipating your desire to add components to the system at a future date, by leaving some inputs unused.  We also offer a one-year Speaker Trade-Up program.

Serviceability

When installing your equipment, we leave extra cable (service loops) in the wiring to allow the equipment to easily be pulled out from the rack for greater accessibility when replacing or servicing the items (unless you have us specify cables of specific lengths).  We leave all owners' manuals and warranty registration cards for the installed equipment for you to complete both for extended warranty protection and for future reference.

Control and Ease of Use

We locate frequently accessed products in the most accessible locations in the Equipment Center.  Our systems are installed to balance the optimum in performance, while providing intuitive control.  Infrared repeater systems, when specified, provide increased flexibility for intuitive use.  The location of equipment and your ability to control it by remote control is no longer dependant on locating it within your line of sight.  An infrared repeater system, allows equipment located in hidden areas, such as in a closed cabinet or even another room, to be controlled by aiming your remote control in the direction of the screen.

Bose - A Commitment to Research

Bose - A Commitment to Reasearch

Since Bose Corporation was founded in 1964 by Massachusetts Institute of Technology Professor Dr. Amar Bose, the original philosophies and founding principles have not changed.

Bose maintains an exceptionally strong commitment to research, for it is within the discipline of research that yesterday's fiction becomes tomorrow's reality. We strive to identify things which, when made better, improve people's lives.

But it's more than just research. We aim for excellence in everything we do. From the way we run our business to our customer service. From the products to the owner's manuals you'll use to set them up. In everything we do, we truly believe that "good enough" is merely a starting point.

We’ve taken our commitment and our passion for innovation and applied it to develop unique sound solutions to meet virtually any audio challenge in any application, even the space program. While many of our products are designed for entertainment and home audio solutions, you’ll find Bose sound is prevalent in both the aviation and automotive industries, too.

We’ve also designed professional sound systems for many applications, including stadiums and auditoriums, houses of worship, retail businesses, department stores and restaurants.

Our commitment has served us well. Today, Bose has operations in the United States, Europe, Canada, Australia and Japan. And we've recently entered the markets in China and India.

Bose Home Theater Speakers

The advent of digital audio and video formats begs for solutions that bring home even more of the enjoyment of music and home theater. Once again, Bose has exercised the spirit of innovation to develop groundbreaking technologies in our Acoustimass® speaker systems.

We've built upon the legacy of these popular systems to deliver a significant level of performance over the full range of music, speech and other effects needed to handle the demands of DVDs and CDs. All from speakers that fit in your hand — and fit nicely with your decor. Whether it's the roar of a runaway train or the sweet song of a cello, you'll enjoy its full measure with the clarity you've come to expect from Bose® speakers.

Monster Cable - Wire it Right...From The Beginning

Whether you’re wiring a single-room A/V system, installing a full blown home theatre system or pre-wiring a house under construction, Monster Cable® speaker cable will fit the bill.

Since labor costs are an overwhelming percentage of any custom installation, it pays to install the best cable possible the first time.

The sonic degradation from conventional cables is significant, especially over long runs common in custom installations.

Monster’s famous cable technologies minimize these distortions for greater clarity, improved dynamic range and smoother sound.

All of Monster’s in-wall speaker cables use ultra-fine, high purity copper stranding for increased flexibility and maximum conductivity and twisted-pair construction to reduce the problems associated with noise and EM/RF interference.

In addition, Monster’s CI speaker cables are built for easy, time-saving installation with such features as an EZ-Strip™ rip cord, low friction jacketing and sequential footage markers printed on the cable jacket.

Don’t compromise on sound. Choose Monster Cable® for the best performance and most reliable custom speaker installation.

Monster Cable Products, Inc. was founded in 1979 by Noel Lee, who was then a laser-fusion design engineer at Lawrence-Livermore Laboratory, as well as an audiophile and professional drummer. Lee discovered that wires of different constructions produced varying degrees of audio performance when hooked up to loudspeakers. From this discovery, he developed a high performance speaker cable, named it Monster Cable, and literally created an industry. The invention of Monster Cable created a new product category that revolutionized the audio market. Prior to Monster Cable, most stereo systems were wired with ordinary "zip-cord," the same cable used for electrical household and lamp wire. Their inferior construction and cheap materials restricts power, dynamic range and clarity in the lows, mids, and highs.

Lee chose the name "Monster" for two reasons. It sounded strong and powerful and the size of the cable was "monsterous" compared to ordinary loudspeaker cable. It became an immediate hit with consumers. In fact, the name Monster Cable is so synonymous with high quality and high performance, customers often assume that any cable they purchase in reputable stores would be Monster even though there are other cable brands.

Monster is now the world's leading manufacturer of high performance cables that connect audio/video components for home, car and professional use as well as computers and computer games. Monster Cable is an indispensable component for music lovers, audiophiles, recording studios, sound professionals, musicians, custom-installers and home theatre enthusiasts. Monster's audio cables increase the clarity, dynamics and power of the audio signals that travel through them. Monster video cables deliver the sharpest, clearest picture possible with naturally vivid color from traditional and digital television, projectors, home theatre and satellite systems. Monster Game products lets enthusiasts enjoy the ultimate gaming experience by maximizing sound and video quality of a gaming system. Monster Power, a division of Monster Cable, delivers cleaner AC power to A/V components and computer products and high joule surge protection for maximum performance.

Monster's New Product Development division draws upon years of audio design and critical listening experience, to create the innovative, high performance cables and accessories that make Monster the leader in the industry. Already holding over 127 US and international patents, Monster is continually striving to discover and develop new, advanced technologies and designs to meet the needs of the ever-advancing consumer electronics industry. Monster offers over 3000 products and is sold in over 80 countries worldwide.